A/ Pixracer basic views

TOP  CONNECTORS

BOTTOM  CONNECTORS

- TOP  CONNECTORS

a/ FRs ( FrSky connector )

- VDD_5V_PERIPH

- FrSky_OUT

- FrSky_IN

- GND

b/ RC ( Receiver connector )

- VDD_5V_RECEIVER

- RC_INPUT ( PPM, SPEKTRUM, SBus )

- RSSI_IN

- VDD_3V3_SPEKTRUM

- GND

c/ TL2 ( Telemetry 2 connector )

- VDD_5V_PERIPH

- FMU_USART3_TxD

- FMU_USART3_RxD

- FMU_USART3_CTS

- FMU_USART3_RTS

- GND

d/ USB

USB connector

e/ TL1 ( Telemetry 1 connector )

- VDD_5V_PERIPH

- FMU_USART2_TxD

- FMU_USART2_RxD

- FMU_USART2_CTS

- FMU_USART2_RTS

- GND

f/ SERVO  CHANNELS

CH1 – CH6 are the servo channels.

NOTE!  The VDD_SERVO rail is NOT connected to the Pixracer power!

Servos require their own 5V BEC!

g/ ESP8266 INTERFACE

- FMU_USART1_RxD

- GND

- ESP8266_PD ( power down )

- ESP8266_GPIO2

- ESP8266_RESET

- ESP8266_GPIO0

- VDD_3V3_PERIPH

- FMU_USART1_TxD

- ESP8266_RTS ( not implemented on ESP-01 module )

- ESP8266_CTS ( not implemented on ESP-01 module )

Pins marked with RED cross are for future use. Do not connect ESP-01 there!

- BOTTOM  CONNECTORS

a/ DEBUG  CONNECTOR

- FMU_VDD_3V3

- FMU_UART7_TxD

- FMU_UART7_RxD

- FMU_SWDIO

- FMU_SWCLK

- GND

b/ PWR ( Power ) CONNECTOR

- POWER_5V_IN

- POWER_5V_IN

- CURRENT_SENSE

- VOLTAGE_SENSE

- GND

- GND

NOTE!  PWR ( POWER SUPPLY) must be between 5.0VDC and 5.5VDC !

Voltages from VOLTAGE and CURRENT sensing should be 3.3VDC max!

c/ BTBZ ( buzzer / button connector )

- SAFETY_SWITCH

- BUT/BUZZ ( button and buzzer common )

- FMU_VDD_3V3

- BUZZER –

- BUZZER +

d/ GPS ( GPS connector )

- VDD_5V_PERIPH

- FMU_USART4_TxD

- FMU_USART4_RxD

- FMU_I2C1_SCL  ( 3.3V tolerant )

- FMU_I2C1_SDA ( 3.3V tolerant )

- GND

- JTAG

The JTAG interface is implemented on a JST SM06B-SRSS 1mm pitch connector.

Very useful device for debugging is the Black Sphere Debugger (http://www.tag-connect.com/BLACK-SPHERE-DBG ).

For flashing the bootloader, firmware and debugging, the cheap STM32 Nucleo or STM LINK-V2 boards can be used (http://mbed.org/platforms/?tvend=10 ).

The Pixracer V1 board is supplied with FMU bootloader flashed, so one doesn’t need to flash it again.

- LEDS and SAFETY SWITCH

There are 2 LEDs on the Pixracer V1 board –Power LED ( GREEN ) and RGB status LED.

LED STATUS TABLE

GREEN = Power LED = always ON

SAFETY SWITCH STATUS

ON = FMU ARMED

SLOW BLINK = Ready to arm

FAST BLINK = Error / Refusing to arm

- POWERING Pixracer V1

There are two power inputs – USB ( only for programming and testing ) and Power from the Power connector.

NOTE!  The Power must be between 5.0VDC to 5.5VDC!

The VDD_SERVO rail is NOT connected to the Pixracer power!

VERY  IMPORTANT  NOTE!

THE  MAX  ALLOWABLE  TOTAL CURRENT  CONSUMPTION  FOR  ALL  PERIPHERALS  IS  1A! Don’t connect the VCC of servos or High Power Telemetry/OSD modules.